TECHNICAL SPECIFICATION

Offline Agentic SOC Assistant

Architecture, Component Specs, Data Schemas, and Implementation Plan

| Field | Value |
| --- | --- |
| Version | 0.1 (Draft) |
| Date | March 2026 |
| Author | Jay |
| Advisor | Dr. Peng Liu |
| Status | Planning / Pre-implementation |

# 1. System Overview

This document defines the technical architecture for an offline, air-gapped SOC assistant built on three open-source components: Kimi K2.5 (reasoning + orchestration), OpenClaw (integration + tool routing), and a local RAG pipeline (threat intelligence). The system processes SIEM alerts through a multi-agent pipeline to produce triage classifications, correlated timelines, and incident response playbooks without any external network dependencies.

## 1.1 Design Principles

- Zero external dependencies after initial deployment. All inference, retrieval, and orchestration happen on-premises.

- Auditable decisions. Every triage output includes a full reasoning chain, confidence score, and evidence trail for compliance review.

- Modular and swappable. Each component (model, agent framework, RAG store, SIEM connector) can be replaced independently.

- Graceful degradation. If the swarm mode hits resource limits, the system falls back to single-agent sequential processing.

- Evaluation-first. The benchmarking harness is built alongside the system, not after it.

## 1.2 Hardware Requirements

| Tier | Hardware | Performance Target | Use Case |
| --- | --- | --- | --- |
| Development | 1x RTX 4090 (24GB) + 64GB RAM | ~5 tok/s (quantized) | Prompt development, pipeline testing |
| Evaluation | 2x A100 80GB + 256GB RAM | ~15-20 tok/s | Full benchmark suite, swarm testing |
| Production | 4x H100 80GB + 512GB RAM | ~40+ tok/s | Real-time SOC deployment |
| Minimum (quantized) | 256GB unified memory (Mac Studio) | ~10 tok/s (INT4) | Air-gapped demo / POC |

All tiers use local NVMe SSD storage for the vector database and model weights. Network connectivity is only required for initial setup (model download, RAG data sync).

# 2. Component Specifications

## 2.1 Data Ingestion Layer

Purpose: Normalize raw SIEM alerts from multiple vendor formats into a unified schema for downstream processing.

Supported Input Formats

| SIEM Platform | Format | Connector Type | Status |
| --- | --- | --- | --- |
| Splunk | JSON via HEC | HTTP webhook listener | Priority 1 |
| Elastic/ELK | JSON via Logstash output | TCP socket listener | Priority 1 |
| QRadar | LEEF over syslog | Syslog UDP/TCP receiver | Priority 2 |
| Generic | CEF over syslog | Syslog parser | Priority 2 |
| File-based | CSV/JSON log files | File watcher (inotify) | For evaluation |

Unified Alert Schema (v1)

{ "alert_id": "uuid-v4", "timestamp": "ISO-8601", "source": "splunk¦elastic¦qradar¦file", "severity": "critical¦high¦medium¦low¦info", "signature": "string (rule/signature name)", "signature_id": "string (vendor rule ID)", "src_ip": "string", "src_port": "integer¦null", "dst_ip": "string", "dst_port": "integer¦null", "protocol": "TCP¦UDP¦ICMP¦other", "raw_log": "string (original log entry)", "metadata": { "vendor": "string", "device": "string", "category": "string (vendor category)" }, "ingested_at": "ISO-8601" }

The ingestion daemon runs as a systemd service, writing normalized alerts to a local Kafka topic (or Redis stream for lighter deployments). Batch mode reads from labeled CSV/JSON files for evaluation replays.

## 2.2 OpenClaw Integration Layer

Purpose: Manages the interface between external systems and the Kimi K2.5 reasoning engine. Handles tool definitions, context window management, session state, and the human-in-the-loop interface.

Key Responsibilities

- Tool registry: defines available tools (SIEM query, RAG search, log lookup, ticket creation) as YAML specs that K2.5 can invoke.

- Context management: sliding window with summarization when conversation exceeds K2.5's effective context. Priority ordering: current alert > recent correlations > RAG context > historical summaries.

- Session state: SQLite-backed session store tracking per-alert triage progress, agent call history, and analyst overrides.

- Rate limiting: controls K2.5 inference requests to prevent GPU saturation during alert storms.

- Interface layer: Telegram bot for analyst interaction, CLI for scripted evaluation, REST API for SOAR integration.

Tool Definition Example

# tools/siem_query.yaml name: siem_query description: > Query the local SIEM index for log entries matching specified criteria within a time window. parameters: query_type: type: string enum: [ip_lookup, signature_search, time_range] value: type: string description: IP address, signature name, or time range time_window_minutes: type: integer default: 30 description: How far back to search (minutes) returns: type: array items: type: object description: Matching log entries in unified schema

Context Priority Stack

| Priority | Content | Max Tokens | Strategy |
| --- | --- | --- | --- |
| 1 (highest) | Current alert + classification request | 2,000 | Always included |
| 2 | Correlated events from Log Correlator | 8,000 | Most recent first |
| 3 | RAG retrieval results | 4,000 | Top-k by relevance |
| 4 | Active session history | 4,000 | Sliding window |
| 5 (lowest) | Historical summaries | 2,000 | Compressed summaries |

Total target: ~20,000 tokens per inference call. K2.5's 256K window provides massive headroom, but keeping prompts focused improves accuracy and reduces latency.

## 2.3 Kimi K2.5 Reasoning Engine

Purpose: Core reasoning, decision-making, and multi-agent orchestration for the entire triage pipeline.

Model Configuration

| Parameter | Value | Notes |
| --- | --- | --- |
| Model | Kimi K2.5 (moonshotai/Kimi-K2.5) | MIT license, open weights |
| Architecture | MoE (1T total, 32B active, 384 experts, 8 per token) | Efficient inference via sparse activation |
| Quantization | Native INT4 (group size 32) | Ships with QAT, no post-hoc accuracy loss |
| Context window | 256K tokens | Sufficient for full incident timelines |
| Serving framework | vLLM or SGLang | Optimized for MoE inference on NVIDIA GPUs |
| Thinking mode | temp=1.0, top_p=0.95, min_p=0.01 | For complex multi-step triage |
| Instant mode | temp=0.6, top_p=0.95 | For simple alert classification |

Operating Modes

The system uses three distinct modes depending on alert complexity:

- Instant Classification: For alerts matching well-known signatures with high confidence. Single inference call, no sub-agents. Target: < 2 seconds.

- Thinking Triage: For ambiguous alerts requiring multi-step reasoning. K2.5 thinking mode with RAG augmentation. Target: < 15 seconds.

- Swarm Investigation: For complex incidents involving multiple correlated alerts. K2.5 decomposes into parallel sub-agents (classifier, correlator, playbook generator). Target: < 60 seconds for full pipeline.

Swarm Task Decomposition

When K2.5 enters swarm mode, it generates a task graph:

SWARM TASK GRAPH (example: suspicious lateral movement) [Orchestrator: Kimi K2.5] ¦ +-- [Sub-agent 1: Alert Classifier] ¦ Input: Original alert (SMB auth failure) ¦ Output: Classification + confidence ¦ +-- [Sub-agent 2: Log Correlator] ¦ Input: src_ip, dst_ip, time_window=30min ¦ Output: Related events timeline ¦ Queries: Auth logs, firewall logs, endpoint logs ¦ +-- [Sub-agent 3: Threat Intel Lookup] ¦ Input: src_ip, signature_id ¦ Output: ATT&CK techniques, known campaigns ¦ Source: Local RAG (MITRE + CVE) ¦ +-- [Sub-agent 4: Playbook Generator] Input: Classification + timeline + threat intel Output: Response playbook Waits for: Sub-agents 1, 2, 3 [Orchestrator aggregates] -> Final triage decision

## 2.4 Agent Specifications

Alert Classifier Agent

| Attribute | Specification |
| --- | --- |
| Trigger | Every incoming normalized alert |
| Output categories | True Positive, False Positive, Needs Investigation, Escalate |
| Confidence threshold | 0.7 (below triggers Log Correlator for additional context) |
| Prompt strategy | Few-shot with 5 examples per alert category, updated monthly |
| Metrics tracked | Per-category precision, recall, F1, false positive rate by signature |

Log Correlator Agent

| Attribute | Specification |
| --- | --- |
| Trigger | Low-confidence classifications (< 0.7) or swarm decomposition |
| Correlation methods | Time-window (±15min), source IP grouping, destination IP grouping, session reconstruction |
| Data sources | Firewall logs, IDS/IPS alerts, authentication logs, endpoint telemetry, DNS queries |
| Output format | Structured JSON timeline with event summaries |
| Context utilization | Leverages K2.5's 256K context for full timeline ingestion |

Playbook Generator Agent

| Attribute | Specification |
| --- | --- |
| Trigger | After classification + correlation complete |
| Framework | NIST SP 800-61 incident response lifecycle |
| Output sections | Summary, Severity Assessment, Containment Steps, Eradication Plan, Recovery Steps, IOC Extraction, Escalation Path |
| Customization | Template library (YAML) for org-specific playbooks |
| RAG integration | Pulls CVE remediation guidance and ATT&CK mitigation recommendations |

## 2.5 Local RAG Pipeline

Purpose: Provides contextual threat intelligence and security knowledge without internet connectivity.

Data Sources

| Source | Update Frequency | Format | Size (approx) |
| --- | --- | --- | --- |
| MITRE ATT&CK v15 | Quarterly (offline sync) | STIX 2.1 JSON | ~50MB |
| NVD/CVE Feed | Weekly (offline sync) | JSON (NVD API format) | ~2GB |
| CIS Benchmarks | Quarterly | PDF (parsed to text) | ~500MB |
| Internal threat reports | As available | Markdown/PDF | Variable |
| NIST SP 800-61 / 800-53 | Annual | PDF (parsed) | ~20MB |

Vector Store Configuration

| Parameter | Value |
| --- | --- |
| Vector database | ChromaDB (development) / Milvus (production) |
| Embedding model | BGE-M3 (local, 567M params) or Nomic Embed v1.5 |
| Chunk size | 512 tokens |
| Chunk overlap | 128 tokens |
| Search strategy | Hybrid (semantic cosine similarity + BM25 keyword) |
| Top-k retrieval | 5 chunks per query (configurable) |
| Distance metric | Cosine similarity, threshold 0.65 |
| Index type | HNSW (ef=200, M=32) |

Offline Sync Protocol

Since the system is air-gapped, RAG data updates follow a manual sync process:

- Download latest ATT&CK/CVE data on an internet-connected workstation.

- Validate checksums and scan for integrity.

- Transfer via encrypted USB or one-way data diode.

- Run the ingestion script which chunks, embeds, and indexes the new data.

- Vector store performs incremental upsert (no full re-index needed).

- Automated smoke test: run 50 known queries and verify retrieval quality.

## 2.6 Output and Decision Layer

Triage Decision Schema

{ "decision_id": "uuid-v4", "alert_id": "reference to original alert", "timestamp": "ISO-8601", "classification": "true_positive¦false_positive¦investigate¦escalate", "confidence": 0.0-1.0, "severity_override": "critical¦high¦medium¦low¦null", "reasoning_chain": [ "Step 1: Alert signature matches known FP pattern...", "Step 2: However, source IP has 3 related alerts in 15min...", "Step 3: RAG confirms technique T1021 (lateral movement)..." ], "correlated_events": ["event_id_1", "event_id_2"], "attck_techniques": ["T1021", "T1078"], "cve_references": ["CVE-2024-XXXXX"], "playbook": { "summary": "string", "containment": ["step 1", "step 2"], "eradication": ["step 1"], "recovery": ["step 1"], "iocs": ["ip:x.x.x.x", "hash:abc123"] }, "processing_mode": "instant¦thinking¦swarm", "latency_ms": 1234, "model_version": "kimi-k2.5-int4", "analyst_override": null, "audit_hash": "sha256 of full decision payload" }

Audit and Compliance

- Every decision written to append-only SQLite database with SHA-256 integrity hash.

- Full reasoning chain preserved for compliance review (NIST 800-53 AU-3 audit content).

- Analyst override capability: any automated decision can be manually changed, with the override logged alongside the original.

- Retention: configurable, default 90 days for decisions, 1 year for audit hashes.

# 3. Data Flow

## 3.1 Real-Time Triage Flow

The primary operational flow for processing a single incoming alert:

- SIEM generates alert and sends to ingestion layer via configured connector.

- Ingestion daemon normalizes the alert to unified schema, assigns UUID, writes to message queue.

- OpenClaw picks up alert from queue, constructs initial prompt with alert data + system instructions.

- OpenClaw sends prompt to Kimi K2.5 in instant mode for initial complexity assessment.

- K2.5 returns complexity score: simple (instant classification), moderate (thinking mode), complex (swarm mode).

- Based on complexity, OpenClaw routes to appropriate processing path.

- For swarm mode: K2.5 generates task graph, OpenClaw dispatches sub-agent tasks in parallel.

- Sub-agents execute (classifier, correlator, playbook generator), each querying RAG as needed.

- Results aggregate back to K2.5 orchestrator for final decision synthesis.

- Final triage decision written to output layer with full audit trail.

- Dashboard updates in real-time. If escalation threshold met, analyst notification triggered.

## 3.2 Evaluation Flow

The benchmarking flow for comparing configurations:

- Load labeled dataset (1,000+ alerts with ground truth classifications).

- Configure test parameters: model, mode (instant/thinking/swarm), RAG on/off, quantization level.

- Replay alerts through the full pipeline in batch mode.

- Capture: classification output, confidence, latency, token count, reasoning chain.

- Compare against ground truth labels. Generate confusion matrix per category.

- Calculate precision, recall, F1, and macro-averaged scores.

- Run statistical significance tests (McNemar's test for paired comparisons, bootstrap CI).

- Generate comparison report with visualizations.

# 4. Evaluation Plan

## 4.1 Benchmark Dataset

| Source | Alert Count | Type | Labels |
| --- | --- | --- | --- |
| CICIDS2017 | ~400 | Network intrusion (DDoS, PortScan, Brute Force, Web Attack, Infiltration) | Ground truth provided |
| BETH Dataset | ~200 | Host-based anomalies (process, syscall) | Ground truth provided |
| Synthetic SIEM | ~300 | Generated from MITRE ATT&CK scenarios | Expert-labeled |
| False Positive Set | ~200 | Known benign alerts from production SIEM patterns | All labeled FP |

Total: 1,100+ alerts across 4 categories (True Positive, False Positive, Needs Investigation, Escalate). Dataset will be released as part of the open-source package.

## 4.2 Comparison Matrix

| Configuration | Model | Mode | RAG | Purpose |
| --- | --- | --- | --- | --- |
| A (baseline) | Kimi K2.5 INT4 local | Single-agent | Off | Baseline local performance |
| B | Kimi K2.5 INT4 local | Single-agent | On | RAG impact measurement |
| C | Kimi K2.5 INT4 local | Swarm | On | Full system performance |
| D | Llama 3.1 70B local | Single-agent | On | Dense model comparison |
| E | GPT-4o via API | Single-agent | On | Cloud baseline |
| F | Claude Sonnet via API | Single-agent | On | Cloud baseline |

## 4.3 Metrics

| Metric | Target | Measurement |
| --- | --- | --- |
| Precision (per category) | > 0.85 | TP / (TP + FP) per class |
| Recall (per category) | > 0.80 | TP / (TP + FN) per class |
| F1 (macro-averaged) | > 0.82 | Harmonic mean across classes |
| False Positive Rate | < 15% | FP / (FP + TN) |
| Latency (instant mode) | < 2 sec | End-to-end per alert |
| Latency (thinking mode) | < 15 sec | End-to-end per alert |
| Latency (swarm mode) | < 60 sec | End-to-end per incident |
| Throughput | > 100 alerts/hour | Sustained processing rate |

# 5. Detailed Implementation Timeline

## Phase 1: Foundation (March 10 – April 30)

Week 1–2: Infrastructure Setup

- Provision GPU hardware. Install CUDA, vLLM/SGLang, Python environment.

- Download Kimi K2.5 INT4 weights. Validate inference with basic prompts.

- Set up OpenClaw on local Node.js environment. Configure basic tool definitions.

- Establish project repository (Git), documentation structure, and CI pipeline.

Week 3–4: Data Pipeline

- Build ingestion daemon with Splunk HEC and ELK Logstash connectors.

- Define and validate unified alert schema. Write schema validation tests.

- Build RAG pipeline: install ChromaDB, download MITRE ATT&CK + CVE data, chunk and embed.

- Curate evaluation dataset: download CICIDS2017, BETH, generate synthetic alerts.

- Label synthetic alerts with ground truth classifications.

Week 5–6: Agent Pipeline v1

- Build Alert Classifier agent with few-shot prompt engineering.

- Build Log Correlator agent with time-window and IP-based correlation.

- Build Playbook Generator agent with NIST 800-61 templates.

- Integrate all agents through OpenClaw tool routing.

- End-to-end smoke test: process 50 alerts through full pipeline.

Phase 1 Milestone Working end-to-end pipeline processing alerts from file input through classification, correlation, and playbook generation. RAG pipeline operational. Evaluation dataset ready.

## Phase 2: Benchmarking (May 1 – June 30)

Week 7–8: Evaluation Harness

- Build automated evaluation framework: batch replay, metric collection, confusion matrix generation.

- Implement statistical testing module (McNemar's, bootstrap CI).

- Run Configuration A (baseline): Kimi K2.5 single-agent, no RAG.

- Run Configuration B: Kimi K2.5 single-agent, with RAG. Measure RAG impact.

Week 9–10: Cloud Comparison

- Set up cloud model API integrations (GPT-4o, Claude Sonnet).

- Run Configurations E and F on identical alert sets.

- Run Configuration D: Llama 3.1 70B local for dense model comparison.

Week 11–12: Swarm Evaluation

- Configure and test K2.5 swarm mode with multi-agent task decomposition.

- Run Configuration C: full swarm with RAG.

- Measure swarm coordination overhead, parallelization gains, and accuracy impact.

- Compile all results into comparative analysis. Run statistical significance tests.

Phase 2 Milestone Complete benchmark results across all 6 configurations. Statistical analysis complete. Clear picture of where local MoE swarm matches, exceeds, or falls short of cloud models.

## Phase 3: Optimization and Paper (July 1 – August 31)

Week 13–14: System Optimization

- Analyze Phase 2 failure cases. Identify systematic weaknesses (e.g., specific alert types, prompt gaps).

- Tune prompts based on error analysis. Update few-shot examples for weak categories.

- Optimize RAG retrieval: adjust chunk size, top-k, similarity threshold based on recall analysis.

- Test sub-agent specialization: fine-tune prompts per agent role.

Week 15–16: Scenario Testing

- Design 5 end-to-end SOC shift scenarios (8 hours each, mixed alert types).

- Run full simulation: streaming alerts, real-time triage, playbook generation.

- Measure analyst time savings estimate based on processing rate and accuracy.

Week 17–20: Paper and Release

- Write research paper. Target: ACM CCS AI & Security Workshop or IEEE CNS.

- Sections: Introduction, Related Work, System Architecture, Methodology, Results, Discussion, Future Work.

- Package open-source release: Docker containers, documentation, evaluation scripts, dataset.

- Internal presentation to Cyber Security Lab.

- Submit paper draft to Dr. Liu for review and iteration.

Phase 3 Milestone Optimized prototype, complete paper draft, and open-source release package ready for submission.

# 6. Risks and Mitigations

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| K2.5 inference too slow on available hardware | Medium | High | Use aggressive INT4 quantization. Fall back to smaller model (Qwen 72B) for initial development. Benchmark on cloud first, optimize locally. |
| Swarm mode unstable or inconsistent | Medium | Medium | Single-agent mode as fallback. Swarm analysis becomes a paper contribution regardless of outcome. |
| Evaluation dataset too small or unrepresentative | Low | High | Supplement with synthetic generation from ATT&CK scenarios. Partner with lab for access to anonymized production data. |
| OpenClaw breaking changes during project | Medium | Medium | Pin specific version. Maintain fork if necessary. Already experienced with version migration. |
| GPU access limited or delayed | Medium | High | Start development on quantized model on available hardware. Request lab GPU allocation early. Cloud GPU as temporary fallback. |

# 7. Deliverables Summary

| Deliverable | Format | Target Date |
| --- | --- | --- |
| Working pipeline (single-agent) | Docker + source code | April 30 |
| Evaluation dataset (1,100+ alerts) | JSON + CSV + labels | April 30 |
| RAG knowledge base | ChromaDB export | April 30 |
| Benchmark results (all configurations) | Report + raw data | June 30 |
| Optimized prototype (swarm mode) | Docker + source code | July 31 |
| Research paper draft | LaTeX + PDF | August 15 |
| Open-source release package | GitHub repository | August 31 |
| Lab presentation | Slides + demo | August 31 |